| Site Information |  |
| --- | --- |
| Parcel Address | 1st Ave South and 11th St North St. Petersburg, FL |
| County | Pinellas |
| Tax Parcel | 24-31-16-53478-000-0210, 24-31-16-53478-000-0300, 24-31-16-53478-000-0211, and a portion of 24-31-16-53478-000-0010 |
| Site Acreage | 2.14 acres |
| Existing Condition of Site | Existing vacant restaurants and associated parking |
| Description of Site Views | Construction bordering left side of site, existing storefront north of site, Tropicana Field parking lot south of site |
| Adjoining Property Uses and Zoning | Zoning: DC-1 Property Uses: Retail |
| Zoning |  |
| Jurisdictions having Authority | City of St. Petersburg |
| Existing Zoning and Comprehensive Plan Designation | Zoning: DC-1, Intown Activity Center |
| Proposed Zoning / Designation | CBD |
| Parking requirements – passenger and loading | Passenger: - Residential, multifamily – 1 per unit >750 sf - Retail Sales & Service – 1 per 500 sf GFA Bike: - Short Term: 2 or 1 per 20 units & 2 or 1 per 5000 sq ft - Long Term: 1 per unit & 2 or 1 per 12,000 sq ft of GFA 1 loading space req for retail operations w/ GFA < 20,000 sq ft 1 loading area required for multi-family developments with more than 5 units |
| Parking stall size requirements – Regular & Compact (% Allowed) | 90° stall: 9’x18’ w/ 24’ drive aisle 45° stall: 9’x19.8’ w/ 13’ drive aisle Compact stall: 8’x18’ (up to 25% of min. req. parking) ADA: 12’x18’ with 5’ access aisle |
| Potential for shared parking | Joint use of required nonresidential parking spaces may occur where two or more uses on the same or separate sites are able to share the same parking spaces because their parking demands occur at different times |
| Setbacks - Parking - Building - Buffers, etc. | Building Setback: - Along street: 0 to 50’ high – 0’ Above 50’ high – 10’ - For buildings that don’t exceed 75’ in height, 10’ setback not required; if building first floor plate <16,000 sq ft 10’ setback not required - Between building: All conditions 50’-200’ – 60’; lots of record with an average lot width ≤ 120 ft., the property shall qualify for a reduced minimum interior building setback from each interior, shared property line. The reduced setback shall be equal to 25% of the lot width, or 15 feet, whichever is greater. |
| Lot requirements - Minimum / Maximum size – Area and dimension - Greenspace, etc. | Minimum Lot Area: no minimum lot area in any DC district Minimum open space: The minimum ground level open space shall be at least five percent of the total land area of the site and 50% pervious. Pedestrian level “B” streets within DC-1: pedestrian oriented uses shall be incorporated into no less than 20 percent of the linear building frontage and have a minimum average depth of 20’ |
| Building size minimums / maximums | Building height: base height 125’, 125’ to 150’ (streamline height) up to 200’ (public hearing) Max. floor plate above 50’: 20,000 sq ft per building |
| Screening requirements | - Where a parking space is designed perpendicular to the street, excluding alleys, such that the front of the space allows the headlights to shine onto the street, a minimum three-foot high solid masonry wall or decorative fence shall be erected to prohibit headlights from shining onto the streets - The exterior of any opaque fence, wall, or dumpster enclosure visible from any street shall be landscaped with a minimum of one shrub for every three linear feet and one under-story tree for every 25 linear feet |
| Potential Variances and Potential for Obtaining |  |
| Maximum Floor Area Requirements for Development | Maximum FAR: 5.0 < FAR ≤ 7.0 (public hearing) Publicly accessible, permitted non residential uses when not located on a pedestrian level “A” street can receive up to 0.5 FAR exemption Limit the building to no more than 50% residential uses, receive 1.5 FAR bonus |
| Requirement for Rezone / PUD | PUD only permitted on sites containing at least 1 acre of contiguous land |
| Landscape Requirements | - Req. green yard depth for all abutting streets (no alleys): 5’ - Min. req. tree landscaping: 1 shade tree per 35 lf - Foundation landscaping required on all building sides visible from street. - POD may reduce required landscape down to 5% where existing site constraints make compliance impractical - Irrigation required for development of multifamily properties |
| Tree Survey / Preservation Requirements | The POD may reduce the required percentage by up to 25 percent to allow preservation of one or more Grand trees that are equal to or greater than the required total inches reduced. 50” or less total in. of ex. trees – 50% of ex. trees to be preserved 51 – 100” total in. of ex. trees – 40% of ex. trees to be preserved 101 -150” total in. of ex. trees – 30% of ex. trees to be preserved |
| Signage Restrictions | Max. # allowed: 1 freestanding sign Max. height: 15 ft Max. sign area: 1 sq ft per linear front foot up to 48 sq ft per sign face Setback: - 0’ if height < 6’ - 3’ if 6’ < height < 10’ - 5’ if 10’ < height < 15’ Construction site, downtown exempt from permitting process but must not exceed 8ft in height No wall sign in DC district and for multi-family residential uses |
| Existing Approvals / PUD on Record with Municipality | N/A |
| Subdivision |  |
| Platting / Subdivision Requirements - Plat or Metes and Bounds | By others |
| Anticipated Takings / Easements that may be Required |  |
| Infrastructure |  |
| Public / Private Utilities | - Water: City of St. Petersburg - Reclaim: City of St. Petersburg - Sanitary Sewer: City of St. Petersburg - Storm: City of St. Petersburg |
| Easements required for utilities | - Stormwater, sanitary, and utility |
| Utility Extensions required | - Potable water |
| Anticipated Roadway Improvements - Lane additions, ROW dedication | - ROW dedication on 1st Ave S - Existing 30’ public ROW (Commercial Ave S) may be dedicated |
| Signalization Required/Potential with Subdivision and Additional GLA | - N/A |
| Potential Encroachments | - Parcel 24-31-16-53478-000-0210 encrouches into alley |
| Additional Access(es) Available that would Improve Traffic Flow to Target and Access Property | - For parcels abutting an alley or secondary street, access shall be from the alley or secondary street. For all other parcels, access shall be from the primary street. |
| Environmental |  |
| Impact Studies Required | - N/A |
| Storm Water Treatment Requirements | - 3.2-inch rainfall in 10 yr/1 hr per COSP master stormwater plan - Post-development runoff rate = pre-development runoff rate |
| Wetlands or Flood Plains Present | - No wetlands, Flood Zone X |
| Setback or Elevation Requirements for Flood Plain/Wetlands | - The lowest habitable floor of all new buildings and structures shall be at least one foot above the average corwn of road measured along building frontage - Not less than a minimum elevation of 103.00 per city datum |
| Natural or Cultural Resources Present on the Site | - Two existing Florida master site files on portion of 24-31-16-53478-000-0010 |
| Environmental Considerations Unique to Area | - Unknown at this time |
| Geotechnical Considerations Unique to Area | - Urban Land |
| Traffic Study Required | - Not included |
| Building |  |
| Current Building Code | - Florida Building Code |
| Unique Construction Methods in the Area (Sinkholes, Piles, Fat Clays, Seismic, Etc.) | - Unknown at this time |
| Fire Route Considerations | - N/A |
| Fees |  |
| Impact/User Fees | Fire Plan Review: $75.00 Building Plan Review: $50.00 Site Plan Review, general app. By POD: $500.00 |
| Application Fees | Building Permit: commercial and all other residential, $750.00 Demolition Permit: over 5,000 sq ft, $250.00 |
| Dedication |  |
| Securities - Landscape, Utilities |  |
| Permit | Building Permit: commercial and all other residential, $750.00 Demolition Permit: over 5,000 sq ft, $250.00 |
| Lot Line Adjustment | Subdivision/replat, preliminary with variances: $1,000.00 Subdivision/replat, final with variances; $1,000.00 |
| Pre-Application Review | $238.00 |
| Coastal Development Permit | N/A |
| Schedule |  |
| Lot line adjustment | Platting can be done at the same time as site plan |
| Entitlements process required for subdivision |  |
| Permitting | - Pre-Application Meeting - Plat Application - Site Plan Review Application with Variance - Neighborhood Noticing Deadline - Tentative DRC Meeting - DRC Approval - Construction Drawings and Building Permit |
| Required municipal staff meetings |  |
| Required municipal public meetings |  |