Table of Contents

GENERAL PROJECT INFORMATION	1

EXISTING CONDITIONS	2

ZONING AND LAND USE	3

PARKING	7

STORMWATER	9

LANDSCAPING	9

UTILITIES	10

PERMITTING	10

Appendix

PROPERTY INFORMATION	APPENDIX A

ZONING INFORMATION	APPENDIX B

EXISTING UTILITY INFORMATION	APPENDIX C

# Site Investigation REPORT

The subject property (“site” or “project”) is generally located northwest of the intersection of 4th Street North and 94th Avenue North in the City of St. Petersburg, Florida as illustrated in the site location map provided by Gensler (Appendix A). The Site is comprised of multiple parcels with a total area of ±66.5 acres.

## GENERAL PROJECT INFORMATION

This project is located within the jurisdiction of City of St. Petersburg. The site currently consists of multiple parcels with a total of ±50.8 acres of buildable land and ±15.72 acres of preserve as shown in the table below.

BAYPOINT COMMERCE CENTER

| Building/Lot Name | Address | Parcel # | Buildable Area (AC) | Preserve Area (AC) | Total Area (AC) |
| --- | --- | --- | --- | --- | --- |
| Baker | 888 Executive Center Dr W | 19-30-17-47069-001-0010 | 5.35 | 6.07 | 11.42 |
| Dade | 9620 Executive Center Dr N | 19-30-17-47063-001-0020 | 2.38 | - | 2.38 |
| Duval | 9450 Koger Blvd | 19-30-17-47062-000-0010 | 2.06 | - | 2.06 |
| Franklin | 701 94th Ave N | 19-30-17-47065-001-0010 | 3.64 | - | 3.64 |
| Gadsden | 9549 Koger Blvd | 19-30-17-47064-003-0010 | 2.18 | - | 2.18 |
| Gilchrist | 801 94th Ave N | 19-30-17-47067-001-0010 | 3.68 | - | 3.68 |
| Glades | 877 Executive Center Dr W | 19-30-17-47060-001-0010 | 6.50 | - | 6.50 |
| Hendry | 9455 Koger Blvd | 19-30-17-47064-003-0012 | 2.48 | - | 2.48 |
| Koger | 9721 Executive Center Dr N | 19-30-17-47063-002-0010 | 3.40 | - | 3.40 |
| Lake | 9400 4th Street N | 19-30-17-47064-003-0013 | 2.74 | - | 2.74 |
| Madison | 9600 Koger Blvd | 19-30-17-47061-001-0012 | 2.07 | - | 2.07 |
| Monroe | 9720 Executive Center Dr W | 19-30-17-47069-001-0020 | 3.00 | 6.71 | 9.71 |
| Pasco | 805 Executive Center Dr W | 19-30-17-47059-001-0010 | 4.41 | 1.34 | 5.75 |
| Pinellas | 9500 Koger Blvd | 19-30-17-47061-001-0010 | 1.76 | - | 1.76 |
| Putnam (vacant lot) | Executive Center Dr N | 19-30-17-47069-002-0010 | 1.44 | 0.52 | 1.96 |
| St. Luicie | 9800 4th Street N | 19-30-17-47068-001-0010 | 3.71 | 1.08 | 4.79 |
| TOTALS |  |  |  |  |  |
|  |  |  | 50.80 | 15.72 | 66.52 |

The proposed project includes the redevelopment of the existing business park with associated parking lots, stormwater management system, and associated utilities.

Project Location:		City of St. Petersburg, FL 33702

Current use:			Business Park

Proposed Use:			Mixed Use

Jurisdictional Authority:		City of St. Petersburg

Proposed Project Area:		±66.52 acres

## EXISTING CONDITIONS

###### Floodplain

According to the Federal Emergency Management Association (FEMA) Flood Insurance Rate Map (FIRM) panel 12103C0207H effective 8/24/2021, the proposed project is located within the Coastal High Hazard Area (CHHA) and Flood Zone AE with a base flood elevation of 9 feet. Refer to the FIRM Panel provided in Appendix A. Any fill within the flood zone will require cup-for-cup floodplain compensation. Development within the CHHA is subject to additional development standards, as outlined in the building requirements section of this report.

###### Wetlands

According to the US Fish and Wildlife Service National Wetlands Inventory, wetlands are present throughout the development area. Refer to the Wetlands Map provided in Appendix A for the approximate location of existing wetlands on site. Impacts to these wetlands may require additional wetland mitigation. A Petition for Formal Determination with the Southwest Florida Water Management District (SWFWMD) will be required to accurately determine the limits of the wetlands present on site.

###### Natural Resource Assessment (NRA)

An NRA is being conducted by Kimley-Horn and Associates, to identify wetlands, surface waters, and upland habitats on-site, including potential sensitive habitats, as well as evaluate the potential for usage by protected species. The NRA is ongoing at the time of this report.

###### Topography

Based on preliminary review of Google Earth and the USGS Topographic Map Viewer, elevations on site generally range from 0ft to 9ft. Refer to the USGS To Map provided in Appendix A. A topographical survey was not available for review at the time of this report and will be required to accurately determine elevations throughout the development.

###### Geotechnical Report and Soils

A Geotechnical Report will be required to determine the characteristics of the soil and ground water table in and around the project area.

According to the hydrologic soil map provided by the United States Department of Agriculture (USDA) Natural Resources Conservation Services, provided in Appendix A, soils on site primarily consist of Basinger fine sand (rating A/D), EauGallie soils and Urban Land (rating A/D), Immokalee soils and Urban Land (rating A/D), Kesson fine sand (rating A/D), Matlacha and St. Augustine soils and Urban Land (rating B), Myakka soils and Urban Land (rating A/D). Group A rated soils are classified as well drained, Group B rated soils are classified as moderately drained, and Group D soils are classified as very poorly drained.

###### Access

Access to the existing business park is provided on 4th St. N., Dr. Martin Luther King Jr. St N., and 94th Ave. N. Currently, the development has one access on Dr. Martin Luther King Jr. St. N, five accesses on 94th Ave. N., and four accesses on 4th St. N. Additionally, access to each of the site parcels is provided on the interior access roads 99th Ave. N., Executive Center Dr. W., Executive Center Dr. N., and Koger Blvd. N. Any access modifications along 4th St. N will require approval through the Florida Department of Transportation (FDOT).

###### Entitlements

Upon review of the boundary surveys prepared by DC Johnson and associates on January 5th, 2006, and the City of St. Petersburg public GIS, a number of electric, drainage, utility, and preservation easements are present throughout the existing business park. Refer to Appendix C for a map of the approximate recorded easement locations.

## ZONING AND LAND USE

The Future Land Use (FLU) designations for the property are Residential/Office General (R/OG) and Planned Redevelopment-Mixed Use (PR-MU) as well as some Transportation/Utility and Preservation designations.  The City has also designated this as a Target Employment Center (TEC) overlay –specifically following the “Gateway Area” policies. The current zoning designations on-site are Corridor Residential Suburban District (CRS-1), Corridor Commercial Suburban District (CCS-1) for the parcels located along 4th Street North, and Preservation (P). A memorandum was prepared by Hill Ward Henderson (HWH), included in Appendix B, to provide insight on the existing zoning and land use entitlements for this development. The development requirements for these sections are discussed below.

###### Building Requirements

Coastal High Hazard Area (CHHA)

The following are development and design regulations within the CHHA, per section 13.30.040 of the City of St. Petersburg (COSP) Land development Code (LDC):

- The following new developments are prohibited in the CHHA: construction of hospitals, nursing homes, assisted living facilities, new mobile home parks, solid waste, and commercial hazardous waste management facilities.

- New construction of residential multifamily developments will require hurricane shelter mitigation, a hurricane evacuation and re-entry plan, and two feet of additional freeboard (for a total of four feet above the minimum FEMA design elevation). In addition, residential multifamily developments containing up to 199 units shall provide one item from the list of development options provided in section 16.30.040.2. Projects containing 200 units or greater shall provide two items from the list and projects containing dwelling units resulting from a density increase from a plan amendment after October 8, 2020 shall provide one additional list item for every 50 additional units.

- Construction, expansion, or substantial renovations of hotel uses will require a hurricane evacuation plan pursuant to the Pinellas County Code, Chapter 34, Article III.

Corridor Residential Suburban District (CRS-1)

The following lots are located within the CRS-1 zone: Baker, Dade, Duval, Franklin, Gadsden, Gilchrist, Glades, Hendry, Koger, Madison, Monroe, Pasco, Pinellas, and Putnam.

Per section 16.20.070 of the COSP LDC, the Building Requirements for the CRS-1 are outlined in Table 1 below:

Table 1: CRS-1 Building Requirements

| Minimum Lot Area (square ft.) |  | 4,500 |
| --- | --- | --- |
| Maximum Impervious Surface Area |  | 0.75 |
| Residential Density (units per acre) |  |  |
| Maximum Density Workforce Housing Bonus |  | 15 6 |
| Non-Residential Intensity (floor area ratio) |  |  |
| Maximum Intensity |  | 0.5 |
| Maximum Building Height (ft) |  |  |
| Primary Building |  | 36 |
| Accessory Building |  | 24 |
| Minimum Building Setbacks (ft) |  |  |
| Front Yard | Stoop | 11 |
|  | Open Porch | 14 |
|  | Building | 20 |
| Side Yard | Interior | 5 |
|  | Street facing | 10 |
| Rear Yard | With Alley | 5 |
|  | Without Alley | 10 |

Note: A 100% intensity bonus is allowed for manufacturing, office, and laboratories and research and development uses on parcels designated as Target Employment Center (TEC) Overlay on the future land use map.

Corridor Commercial Suburban District (CCS-1)

The following lots are located within the CCS-1 Zone: Lake (large lot) and St. Luicie (medium lot).

Per section 16.20.090 of the COSP LDC, the Building Requirements for the CCS-1 are outlined in Table 2 below:

Table 2: CCS-1 Building Requirements

| Minimum Lot Area (square ft.) |  |  |  |  | 4,500 |
| --- | --- | --- | --- | --- | --- |
| Maximum Impervious Surface Area |  |  |  |  | 0.85 |
| Residential Density (units per acre) |  |  |  |  |  |
| Maximum Density Workforce Housing Bonus |  |  |  |  | 15 8 |
| Hotel Density |  |  |  |  | 45 |
| Non-Residential Intensity (floor area ratio) |  |  |  |  |  |
| Maximum Intensity Workforce Housing Bonus |  |  |  |  | 0.55 0.2 |
| Maximum Building Height (ft) |  |  |  |  |  |
|  | Small Lot (< 1.0 acres) | Medium Lot (1.0 – 2.0 acres) |  | Large Lot (> 2.0 acres) |  |
| All Buildings | 36 | 36 |  | 48 |  |
| Minimum Building Setbacks (ft) |  |  |  |  |  |
|  | Small Lot (< 1.0 acres) |  | Medium Lot (1.0 – 2.0 acres) |  | Large Lot (> 2.0 acres) |
| Adjacent to Street (not alley) Residential Non-Residential | 0 - 30 10 - 30 |  | 20 - 80 20 - 80 |  | 20 20 |
| Interior Side Yard Non-Residential Abutting Non-Residential Non-Residential Abutting Residential Residential Abutting Residential | 10 25 20 |  | 10 35 20 |  | 10 50 20 |
| Rear Yard Non-Residential Abutting Non-Residential Non-Residential Abutting Residential Residential Abutting Residential | 20 25 20 |  | 20 35 20 |  | 20 50 20 |

## parking

###### Vehicular Parking Spaces

Per section 16.10.020.1 of the COSP LDC, the minimum number of parking spaces required are outlined in Table 3 below:

Table 3: Minimum Parking Requirements

| Proposed Use | Minimum Parking Spaces Required |  |
| --- | --- | --- |
| Multifamily Residential | Equal to or less than 750 square feet | 0.75 per unit |
|  | More than 750 square feet | 1.25 per unit up to 2 bedrooms, plus 0.5 for each additional bedroom |
| Hotel | 1 per room |  |
| General Office | 1 per 300 SF GFA |  |
| Retail | 1 per 300 SF GFA |  |

Compact parking spaces may be provided for up to 25 percent of the minimum required parking for all uses. Where a particular use may have a low turn-over parking ratio, such as a corporate or industrial land use, up to 100 percent compact parking, excluding required accessible parking, may be approved. Compact parking spaces shall be identified or designated as such through the use of signage or pavement markings.

The minimum number of required accessible parking spaces are outlined in Table 4 below:

Table 4: Required Accessible Spaces

| Total Parking Provided | Required Number of Accessible Spaces |
| --- | --- |
| Up to 25 | 1 |
| 26 to 50 | 2 |
| 51 to 75 | 3 |
| 76 to 100 | 4 |
| 101 to 150 | 5 |
| 151 to 200 | 6 |
| 201 to 300 | 7 |
| 301 to 400 | 8 |
| 401 to 500 | 9 |
| 501 to 1,000 | 2 percent of total |
| Over 1,000 | 2 percent of total, plus 1 for each 100 over 1,000 |

Dimensions

Per section 16.40.090.3.4 of the COSP LDC, the dimensions for standard, compact, and accessible parking spaces are outlined in Table 5 below:

Table 5: Parking Stall Dimensions

| Parking Stall Type | Parking Stall Dimension |
| --- | --- |
| Standard | 9’ x 18’ |
| Compact | 8’ x 18’ |
| Accessible | 12’ x 18’ with a 5’ access aisle |

###### Bicycle Parking Spaces

The minimum number of required bicycle parking spaces are outlined in Table 6 below. Buildings with less than 2,500 square feet of gross floor area are not required to provide long-term bicycle parking. Payment in lieu of required bicycle parking is allowed at a rate of $83.00 per space not provided, however new construction must provide all required bicycle parking and may not make a payment in lieu of required parking without a variance.

Table 6: Required Bicycle Spaces

| Bicycle Parking Spaces Required |  |  |
| --- | --- | --- |
| Use Category | Short-Term Spaces | Long-Term Spaces |
| Residential, multifamily | 2, or 1 per 20 units | 1 per each unit |
| Residential, dormitory | 1 per 20 bedrooms | 1 per 2 bedrooms |
| Retail sales | 2, or 1 per 5,000 sq. ft. | 2, or 1 per 12,000 sq. ft. of gross floor area |
| Restaurant | 2, or 1 per 5,000 sq. ft. of gross floor area | 2, or 1 per 10,000 sq. ft. of gross floor area |
| Office | 2, or 1 per 40,000 sq. ft. of gross floor area | 2, or 1 per 10,000 sq. ft. of gross floor area |

Loading Spaces

Per section 16.40.090.5.1 of the COSP LDC, the minimum number of loading spaces required shall be calculated from Table 7 below:

###### Table 7: Required Loading Spaces

| ###### Use Category | ###### Requirements |
| --- | --- |
| ###### Retail operations and all first-floor nonresidential uses with a gross floor area of up to 20,000 sq. ft. | ###### 1 space |
| ###### Retail operations and all first-floor nonresidential uses with a gross floor area of more than 20,000 sq. ft up to 50,000 | ###### 1 berth for local delivery trucks |
| ###### Retail operations and all first-floor nonresidential uses with a gross floor area of more than 50,000 sq. ft. | ###### 1 berth for local delivery trucks, 1 berth for semi-trailers |
| ###### Office buildings and hotels with a gross floor area of 100,000 sq. ft. or more | ###### 1 berth for semi-trailers for every 100,000 sq. ft. of gross floor area |

## stormwater

Any proposed development or redevelopment of the project area will require stormwater treatment and attenuation as outlined below. The project area currently discharges to Tinney Creek which has a known impairment for dissolved oxygen, therefore additional treatment beyond the requirements outlined below will be required to achieve net improvement for development.

The following parcels fall within existing SWFWMD Environmental Resource Permits (ERP): Putnam vacant lot (5424.002), St. Luicie lot (4411.000), COSP Tinney creek (18139.000), Glades lot (1824.000), Pasco lot (5424.001). Further development or redevelopment of these parcels will require a modification to the existing ERP. For the remaining lots that do not fall within an existing ERP, further development or redevelopment will require either a full ERP or an exemption. Guidelines for ERP’s are outlined in the permitting section of this report.

Southwest Florida Water Management District

The Southwest Florida Water Management District (SWFWMD) requires treatment and attenuation of stormwater discharge generated by development. The post-development discharge rate from the site is limited to equal to or less than the discharge rate in the pre-development condition for the 25 year-24-hour design storm. In addition, stormwater treatment is required for the first one-half inch of rainfall over the site for dry retention and one inch for wet detention.

City of St. Petersburg

The City of St. Petersburg requires any redevelopment to provide treatment for the first one-half inch of rainfall over the site for dry retention across the disturbed areas. The post-development discharge rate must be less than or equal to the predevelopment rate for the 10 year-1 hour design storm.

## LANDSCAPING

###### Vehicular Use Areas

Per Section 16.40.060.2.1.3 of the COSP LDC, parking lots or portions of parking lots not visible from the street, excluding alleys, are not required to install perimeter landscaping. Where a parking space is designed perpendicular to the street, excluding alleys, such that the front of the space allows the headlights to shine onto the street, a minimum three-foot high solid masonry wall or decorative fence shall be erected to prohibit headlights from shining onto the streets.

For all vehicular use areas with more than ten parking spaces, a minimum of ten percent of the vehicular use area shall be devoted to interior landscaping. Each interior island shall measure at least eight feet in width by 18 feet in length, measured from the inside of the curb. Within interior islands, one shade tree shall be required for every 150 square feet with a minimum of one shade tree required per interior island. Landscaping in islands adjacent to parking spaces shall be set back a minimum of two feet behind the back of the curb to provide for pedestrian access to parked vehicles.

###### Foundation Landscaping

Per Section 16.40.060.2.1.3 of the COSP LDC, the required green yard depth for all abutting streets (not alleys) is ten (10) feet. Additionally, the minimum required tree landscaping is two (2) shade trees per 50 linear feet or fraction above half thereof. A minimum of one foundation plant is required for each three linear feet, and one under-story tree is required for each 30 linear feet (or portion thereof), of the exterior building perimeter. Foundation plantings may be comprised of shrubs, accent plants, ornamental grasses, and ground cover in any combination; provided that no less than 50 percent of the total required materials are shrubs, accent plants and/or ornamental grasses.

###### Tree Removal

Per Section 16.70.030.1.12 of the COSP LDC, a Tree Removal Permit is required for the removal of any protected tree or palm. At the time of this report, a tree survey was not available.

## UTILITIES

###### Potable Water

Potable water is served by the City of St. Petersburg and is currently available throughout the business park. Utility atlas maps have been requested from the City.

###### Sanitary Sewer

Sanitary sewer is served by the City of St. Petersburg and is currently available throughout the business park. Utility atlas maps have been requested from the City.

###### Reclaim Water

Reclaimed water is served by the City of St. Petersburg and is currently available throughout the business park. Utility atlas maps have been requested from the City.

###### Gas

Gas is served by TECO Peoples Gas. Utility maps have been requested from TECO.

###### Electric

Electric service is provided by Duke Energy. Utility maps have been requested from Duke.

## PERMITTING

###### City of St. Petersburg

- Pre-Application Meeting will be required.

- Site Plan Review Application

  - Site Plan Review Application with no Variances

    - Streamline process (±2-3 months)

  - Site Plan Review Application with Variances

    - Neighborhood Noticing

    - DRC Meeting

    - DRC Approval

- Construction Drawings and Building Permit

  - ±3-6 months after DRC approval depending on number of rounds of comments.

Note: Tree Permit and Sign Permit Submitted concurrent with Site Permit.

###### Southwest Florida Water Management District (SWFWMD)

If a development does not qualify for an exemption as outlined below, a full ERP or ERP modification will be required. SWFWMD permitting can run concurrently with COSP permitting.

Exemption: ±2-3 months

- Developments that do not result in any of the following qualify for an exemption.

- Any project in, on, or over wetlands or other surface waters

- A total of more than 4,000 square feet of impervious and semi-impervious surface areas subject to vehicular traffic

- A total of more than 9,000 square feet of impervious and semi-impervious surface area

- A total project area of more than five acres

- A capability of impounding more than 40 acre-feet of water

- Any project that is part of a larger common plan of development or sale

- Any modification or alteration of a project previously permitted.

Environmental Resource Permit (ERP): ±4-6 months

- Not qualifying for exemption and not included in an existing ERP.

ERP Modification

- Minor modifications: ±3-5 months

- To transfer a permit upon a change in ownership or control

- To make other minor changes that do not substantially alter the permit authorization, increase permitted off-site discharge, increase the environmental impact of the project, decrease required retention, decrease required detention, decrease required flood control elevations, or decrease pollution removal efficiency.

- Major modifications: ±4-6 months

- Any application for modification that does not qualify for a minor modification as described above shall be processed as a major modification.

###### Florida Department of Environmental Protection (FDEP)

A Notice of Intent to Use the General Permit for Construction of Water Main Exemptions permit will be required through FDEP for the watermain extension. A Notification/Application for Construction a Domestic Wastewater Collection/Transmission System will be required for the sanitary main extension. Approximately ±2 months for review. Can run concurrent with City of St. Petersburg Permitting and SWFWMD Permitting. Lastly, a National Pollutant Discharge Elimination System (NPDES) exemption will be required for stormwater discharge.